### EarthBound Mulch Tackifier

Specifications

THE SPECIFICATIONS DESCRIBED WITHIN IS AN ATTEMPT TO COVER MOST DESIGN ASPECTS OF UTILIZING THE EARTHBOUND. THIS DOCUMENT IS INTENDED TO BE UTILIZED AS A GUIDELINE, WHICH CAN BE CUSTOMIZED BY THE USER TO MEET SPECIFIC PROJECT REQUIREMENTS.

THIS PROVISION DOES NOT ATTEMPT TO ADDRESS ALL ASPECTS OF EARTHBOUND DESIGN. A PROFESSIONAL ENGINEER RESPONSIBLE FOR DESIGN SHOULD CHECK THE INFORMATION PRESENTED IN DETAIL. CONSULT YOUR EARTHBOUND REPRESENTATIVE FOR MORE INFORMATION.

CONTACT YOUR EARTHBOUND REPRESENTATIVE AT (888) 843-1029 FOR MORE SPECIFIC RATE RECOMMENDATIONS BASED ON SOIL TYPE, SLOPE AND OTHER CONDITIONS.

® EarthBound is a registered trademark of Terra Novo Inc

### EarthBound Mulch Tackifier

### Specifications

1.  GENERAL

1.1.  Work Included.  EarthBound shall be furnished and applied as shown on the drawings and as required for proper installation and functioning of the product.

2.  MATERIALS

2.1. EarthBound Mulch Tackifier. The mulch tackifier to be applied shall be “EarthBound” as manufactured by Terra Novo Inc (888) 843-1029. Always close or seal containers after use. Store in a dry, insulated location to avoid moisture and freezing. EarthBound Mulch Tackifier shall be supplied in liquid or in granular form.

2.1.1  EarthBound L Liquid Mulch Tackifier. EarthBound L Liquid Mulch Tackifier shall be 30% active anionic polyacrylamide and contains less than 500 ppm residual acrylamide ANSI/NSF Standard 60.

2.1.2  EarthBound Dry Mulch Tackifier. EarthBound Mulch Tackifier shall be granular anionic polyacrylamide and contains less than 500 ppm residual acrylamide ANSI/NSF Standard 60.

- 2.2.  Mulch.  The mulch to be used shall be either of wood fiber, paper fiber or a combination of the two and shall be produced from natural or recycled (pulp) fiber, such as wood chips or similar wood materials or from newsprint, and shall be free of synthetic or plastic materials.

Fiber shall be of such character that the fiber will disperse into uniform slurry when mixed with water. Fiber shall be colored to contrast with the area on which the fiber is to be applied, and shall not stain concrete or painted surfaces.

- A Certificate of Compliance for fiber shall be furnished to the Engineer.

2.3  Seed and Amendments.

<<<<<<INSERT SPECIFICATION>>>>>

3.  EARTHBOUND APPLICATION AND RATES

EarthBound shall be applied in combination with water and mulch, mixed in the following proportions. Consult your EarthBound Sales Representative and/or product manufacturer Terra Novo Inc at (888) 843-1029 for the appropriate project specific application rates.

Fill hydroseeding vessel with water to at least 1/3 capacity (high enough to cover agitators) prior to adding any EarthBound. Continue to fill vessel with water and slowly add the EarthBound while agitators are in motion. Once full with the accurate amount of EarthBound in solution, add mulch, and seed and amendments. Continue to mix tank a minimum of 10 minutes prior to application.

Spray in a manner to ensure complete and proper coverage. Minimal disturbance of treated areas after application is recommended.

A Certificate of Compliance shall be furnished to the Engineer stating that the appropriate quantities, blending and application methods as specified here within have been properly followed.

3.1.1  EarthBound L Mulch Stabilizer.

| SLOPE | 6:1 | 5:1 | 4:1 | 3:1 | 2:1 |
| --- | --- | --- | --- | --- | --- |
| EARTHBOUND (gal/ac) | 1 | 1 | 2 | 2 | 2 |
| MULCH (lb/ac) | 1,500 | 1,500 | 1,500 | 1,800 | 2,000 |
| SEED and AMENDMENTS (lb/ac) |  |  |  |  |  |

3.1.2  EarthBound Dry Mulch Stabilizer.

| SLOPE | 6:1 | 5:1 | 4:1 | 3:1 | 2:1 |
| --- | --- | --- | --- | --- | --- |
| EARTHBOUND (lb/ac) | 3 | 3 | 4 | 5 | 5 |
| MULCH (lb/ac) | 1,500 | 1,500 | 1,500 | 1,800 | 2,000 |
| SEED and AMENDMENTS (lb/ac) |  |  |  |  |  |

4.  REPAIR OF DAMAGED AREA

Any damaged area shall be repaired utilizing the exact blend and application procedure as specified above.

5.  PAYMENT

- The EarthBound application will be paid for by the square foot of area treated. The price shall include full compensation furnishing all labor, materials, tools, equipment, and incidentals, and for doing all EarthBound installation, complete in place, as shown on the plans, and as specified in these special provisions, and as directed by the Engineer.